SPECIFICATIONS SECTION 07720

RIDGE VENT SPECIFICATION

# I. 				PART ONE - GENERAL

1.1		SUMMARY

A.	Work included: Furnishing and installing factory fabricated ridge vents.

B.	Related Work: [Insert applicable specifications sections]

1. 2 		REFERENCES

A.	American Society for Testing and Materials (ASTM), 100 Bar Harbor Drive, West Conshocken, PA 19428; (610) 832-9585, fax (610) 832-9555

1.3 		SUBMITTALS

A.	Product Data: Provide manufacturer’s product data for all materials in this specification.

B.	Shop Drawings: Show profiles, accessories, location, adjacent construction interface, and dimensions.

C.	Samples: Manufacturer to provide upon request; sized to represent material adequately.

D.	Contract Closeout: Ridge vent manufacturer shall provide the manufacturer’s Warranty prior to the contract closeout [if applicable].

1.4 		PRODUCT HANDLING

A.	All materials shall be delivered in manufacturer’s original packaging.

B.	Store materials in a dry, protected, well-vented area. The contractor shall thoroughly inspect product upon receipt and report damaged material immediately to delivering carrier and note such damage on the carrier’s freight bill of lading.

1.5			SUBSTITUTIONS

A.	Proposals for substitution products shall be accepted only from bidding contractors and not less than (10) working days before bid due date. Contractor guarantees that proposed substitution shall meet the performance and quality standards of this specification.

1.6			JOB CONDITIONS

A.	Verify that other trades with related work are complete before installing roof ridge vent(s).

B.	Mounting surfaces shall be straight and secure; substrates shall be of proper width.

C.	Refer to the construction documents, shop drawings, and manufacturer’s installation instructions.

D.	Coordinate installation with roof sheeting and roof insulation manufacturer’s instructions before starting.

E.	Observe all appropriate OSHA safety guidelines for this work.

1.7 		WARRANTY/GUARANTEE

A.		Manufacturer’s standard warranty: Materials shall be free of defects in material and workmanship for a period of one year from the date of purchase. Should a part fail to function in normal use within this period, manufacturer shall furnish a new part at no charge. Electrical motors, special finishes, and other special equipment (if applicable) shall be warranted separately by the manufacturers of those products.

II. 		PART TWO - PRODUCTS

2.1		MANUFACTURER

- R & S Manufacturing and Sales Company, Inc., 3575 Old Conejo Rd., Newbury Park, CA 91320

1-805-375-3980, Fax: 1-805-375-3981,Web: www.rnssales.com

2.2		RIDGE VENT

A.	Furnish and install where indicated on plans metal ridge vent Type RV size: [insert: throat width (______)] x 10’ length. The roof ridge vent shall be preassembled from the manufacturer.

B.	Performance characteristics:

1.	   Lifting mechanism assembly shall open the vent damper (when supplied) when either the chain is released from the keeper or when the pull bar is drawn to a closed position via cables or hand crank(s).  Damper shall open and close in a vertical direction and controled with a guide rod.

- 2.	    Entire ridge vent shall be weather resistant with a top diverter, end caps, and wind bands to direct 						    rain away from the vent opening.

- Latch mechanisms or hand cranks shall hold the damper(s) in the closed position.

- Screening shall be #2 mesh 19 GA galvanized steel screen and is placed along the top opening of the vent.

- Venting performance shall be as listed on the manufacturer’s spec sheet and shall be based on providing intake air with the equivalent of 1-1/2 times the ventilator open area.

- Construction: Windbands, skirts, diverters, and end caps shall be minimum 26 GA galvanized or zincalume steel for RV9 or RV12 Models.  RV18 and RV24 Models shall be constructed with a minimum 22 GA A653 galvanized or A792 zincalume steel.    												1.	Internal gussets shall be a minimum 22 GA A653 galvanized or A463 aluminized steel.  For                           	    RV18 and RV24 Models, internal gussets shall be a minimum 18 GA A653 galvanized or A463 	               	    aluminized steel.                                                                                                                     	                  	2. Internal braces and damper operator arms shall be a minimum 18 GA A653 galvanized or A463         	     aluminized steel.  For RV18 and RV24 Models, internal braces shall be a minimum 14 GA A653 	     	     galvanized or A463 aluminized steel.

D.	Finish: External visible parts shall be either mill finish,  a pre-coated SMP off white finish, or a custom color powder coat finish.  Internal braces, operator parts, bird screen, or trim shall be mill finish.

III. 		PART THREE - EXECUTION

3.1 	INSPECTION

A.	    Verify that ridge vent installation will not disrupt other trades. Verify that the substrate is dry, clean, and free of foreign matter. Report and correct defects prior to any installation.

3.2 	INSTALLATION

A.	    Submit product design drawings for review and approval to the architect or specifier before fabrication.

B.	   The installer shall check as-built conditions and verify the manufacturer’s roof fire vent details for accuracy to fit the application prior to fabrication. The installer shall comply with the ridge vent manufacturer’s installation instructions.

C.    Ridge vents are installed as single units or as continuous vents with multiple vent flashings.

D.    Ridge vents should not be installed closer than 5’0” to the endwall.

E.	    The installer shall furnish mechanical fasteners, sealants and closurers consistent with the roof 								 requirements.

F.     The end cap flashing may be field cut to match roof pitches up to 6:12.

G.	  The installer shall test the vent(s) damper(s) for proper operation.

  - MAINTENANCE

- Test each damper at least annually. Lubricate pulleys and hinges annually.

- Inspect  metal surfaces for dirt and corrosion every six (6) months or more often as conditions warrant.

Clean and touch up painted or coated surfaces as required.

END OF SECTION 07720